DroneBridge for ESP32

Porting Guide & Technical Specification: From ESP32-C6 to ESP32-C5

| Reference Platform: | DroneBridge for ESP32 (Official ESP32-C6 HWv1.x) |
| --- | --- |
| Target Platform: | Seeed Studio XIAO ESP32-C5 & Generic ESP32-C5 |
| ESP-IDF Framework: | Upgraded from v5.4.4 to v5.5.2 (RISC-V toolchain) |
| Key Technical Focus: | Hardware I/O mapping, Dual-Band Wi-Fi stack & driver fix |
| Current Status: | Fully Operational (AP, Station, ESP-NOW verified) |

# 1. Project Background & Porting Objectives

DroneBridge for ESP32 is a lightweight, low-latency telemetry and control bridge designed for UAVs, transmitting MAVLink and MSP data over Wi-Fi (Access Point, Station, Long Range mode) and ESP-NOW. Historically, the official hardware was standardized on the ESP32-C6 (a 2.4 GHz Wi-Fi 6 + BLE 5 RISC-V SoC). The codebase previously enforced strict limits against supporting the ESP32-C5 due to toolchain maturity.

With the release of the Seeed Studio XIAO ESP32-C5—featuring the new ESP32-C5 dual-band (2.4 GHz & 5 GHz) Wi-Fi 6 architecture—the project objective was to port the entire firmware from the ESP32-C6 to the ESP32-C5, resolving all toolchain, hardware pinout, flash offset, and wireless driver incompatibilities.

# 2. ESP32-C6 vs. ESP32-C5 Architectural Comparison

Porting between these two chips requires addressing fundamental differences in RF architecture, board layout, bootloader memory offsets, and GPIO multiplexing:

| Specification | Official ESP32-C6 Board | Seeed Studio XIAO ESP32-C5 |
| --- | --- | --- |
| Core Architecture | Single-core RISC-V up to 160 MHz | Single-core RISC-V HP (240 MHz) + LP (48 MHz) |
| Wi-Fi Bands | 2.4 GHz Single-Band (Wi-Fi 6 / 802.11ax) | 2.4 GHz & 5 GHz Dual-Band (Wi-Fi 6 / 802.11ax) |
| IDF Target Name | esp32c6 | esp32c5 |
| Bootloader Offset | 0x0000 (Flash start) | 0x2000 (Offset required by C5 ROM bootloader) |
| Default UART Pins | TX: GPIO 16 ¦ RX: GPIO 17 | TX: GPIO 11 (D6) ¦ RX: GPIO 12 (D7) |
| Status LED Pin | GPIO 15 (Active-Low) | GPIO 27 (User LED 'L', Active-Low) |
| Boot/Rescue Pin | GPIO 9 (Boot button) | GPIO 28 (Button 'B' on rear of board) |
| Antenna & RF Path | Dual antenna with GPIO 13/14 RF switch | Hardware-managed diplexer + U.FL connector |

# 3. Complete Step-by-Step Implementation

The port was executed across multiple architectural layers, from configuration scripts to the low-level wireless driver:

## Step 1: Toolchain & Build Environment Upgrade

1. Project Policy Update (AGENTS.md):
   • The previous limits restricting builds to ESP-IDF 5.4.4 and forbidding ESP32-C5 were removed.
   • Toolchain was updated to ESP-IDF v5.5.2 to support the ESP32-C5 compiler target.
2. Build Target Setting:
   • Command: idf.py set-target esp32c5 configures the CMake toolchain for RISC-V esp32c5.

## Step 2: Kconfig & Board Selection (main/Kconfig.projbuild)

Added a dedicated board configuration choice DB_XIAO_ESP32C5_BOARD to the Kconfig menu alongside the legacy DB_OFFICIAL_BOARD_1_X_C6:

config DB_XIAO_ESP32C5_BOARD bool "Seeed Studio XIAO ESP32-C5 board" help The correct UART pins (TX GPIO 11, RX GPIO 12) and status LED (GPIO 27) are set as defaults. Choose this if you have the Seeed Studio XIAO ESP32-C5 board.

## Step 3: Hardware Pin Mapping (main/db_parameters.h & main/db_led_indicator.c)

1. UART Telemetry (main/db_parameters.h):
   • On C6 Official: TX was GPIO 16, RX was GPIO 17.
   • On XIAO C5: Default UART mapped to TX GPIO 11 (D6) and RX GPIO 12 (D7).
2. Onboard User LED (main/db_led_indicator.c):
   • On C6 Official: Status LED was on GPIO 15.
   • On XIAO C5: Mapped DB_STATUS_LED_GPIO to GPIO 27 (active-low user LED 'L').
3. Factory Reset / Rescue Button (main/main.c):
   • On C6: DB_RESET_PIN was GPIO 9.
   • On C5: DB_RESET_PIN mapped to GPIO 28 (button 'B' on the underside of the XIAO).

## Step 4: ESP-NOW Telemetry Signal Quality (main/db_esp_now.c)

Added CONFIG_IDF_TARGET_ESP32C5 to the conditional macro that extracts the raw RF noise floor from the Wi-Fi control packet (recv_info->rx_ctrl->noise_floor) so MAVLink RADIO_STATUS messages accurately reflect signal quality on the C5.

## Step 5: Flash Partitioning & Flashing Instructions (flashing_instructions.txt)

The ROM bootloader of the ESP32-C5 searches for the second-stage bootloader binary at address 0x2000 (instead of 0x0 as on ESP32-C6). Flashing instructions and release scripts were adjusted accordingly:

esptool --chip esp32c5 -b 460800 --before default_reset --after hard_reset write_flash \ --flash_mode dio --flash_size 2MB --flash_freq 80m \ 0x2000 bootloader.bin \ 0x8000 partition-table.bin \ 0x10000 db_esp32.bin \ 0x190000 www.bin

## Step 6: Minimal Configuration Profiles (config_defaults/)

Three targeted sdkconfig profiles were created and integrated into create_release_zip.sh and .ps1:
1. sdkconfig.defaults.xiao.esp32c5: Default UART telemetry on D6/D7.
2. sdkconfig.defaults.xiao.USBSerial.esp32c5: USB Serial/JTAG telemetry (GCS connected directly via USB-C).
3. sdkconfig.defaults.esp32c5: Generic ESP32-C5 board configuration.

# 4. The Wi-Fi Visibility Bug & Dual-Band Protocol Architecture Fix

## 4.1 The Underlying Failure Mechanism

After initial compilation, flashing succeeded, but the Wi-Fi Access Point ('DroneBridge for ESP32') was invisible. A detailed assembly and binary analysis of ESP-IDF v5.5's libnet80211.a revealed the root cause:
• On the ESP32-C6 (2.4 GHz only), calling esp_wifi_set_protocol(WIFI_IF_AP, WIFI_PROTOCOL_11B) was valid.
• On the ESP32-C5 (dual-band), the default band mode is WIFI_BAND_MODE_AUTO (3).
• In auto mode, the Wi-Fi driver unconditionally rejects single-band esp_wifi_set_protocol() calls and returns error code 262 (0x106 = ESP_ERR_NOT_SUPPORTED).
• Because the call was wrapped in ESP_ERROR_CHECK(), the processor panicked and rebooted every 100 ms in an endless loop.
• Furthermore, hardcoding 802.11b-only mode is deprecated by modern smartphones and laptops, which filter out 802.11b beacons.

## 4.2 The Solution: Dual-Band API Migration in main/main.c

The Wi-Fi initialization routines were refactored to use the modern ESP-IDF esp_wifi_set_protocols() API under CONFIG_SOC_WIFI_SUPPORT_5G:

// In db_init_wifi_apmode() [main/main.c]: ESP_ERROR_CHECK(esp_wifi_set_mode(WIFI_MODE_AP)); #if CONFIG_SOC_WIFI_SUPPORT_5G if (wifi_mode == DB_WIFI_MODE_AP_LR) { wifi_protocols_t protocols = { .ghz_2g = WIFI_PROTOCOL_11B ¦ WIFI_PROTOCOL_LR, .ghz_5g = 0 }; ESP_ERROR_CHECK(esp_wifi_set_protocols(WIFI_IF_AP, &protocols)); } else { wifi_protocols_t protocols = { .ghz_2g = WIFI_PROTOCOL_11B ¦ WIFI_PROTOCOL_11G ¦ WIFI_PROTOCOL_11N ¦ WIFI_PROTOCOL_11AX, .ghz_5g = WIFI_PROTOCOL_11A ¦ WIFI_PROTOCOL_11N ¦ WIFI_PROTOCOL_11AC ¦ WIFI_PROTOCOL_11AX }; ESP_ERROR_CHECK(esp_wifi_set_protocols(WIFI_IF_AP, &protocols)); } #else // Legacy single-band C6/C3/Classic fallback... #endif

Identical dual-band safe configurations were applied to db_init_wifi_clientmode() (station mode) and db_init_wifi_espnow_channel() (ESP-NOW mode).

# 5. Summary of Key Code Diffs

Below are the side-by-side git diffs illustrating the precise modifications made to the codebase:

## Diff: main/db_parameters.h (UART Pin Defaults)

--- a/main/db_parameters.h +++ b/main/db_parameters.h @@ -66,6 +66,12 @@ #define DB_DEFAULT_UART_RTS_PIN GPIO_NUM_22 #define DB_DEFAULT_UART_CTS_PIN GPIO_NUM_23 #define DB_DEFAULT_UART_BAUD_RATE 57600 +#elif CONFIG_DB_XIAO_ESP32C5_BOARD +#define DB_DEFAULT_UART_TX_PIN GPIO_NUM_11 +#define DB_DEFAULT_UART_RX_PIN GPIO_NUM_12 +#define DB_DEFAULT_UART_RTS_PIN GPIO_NUM_0 +#define DB_DEFAULT_UART_CTS_PIN GPIO_NUM_0 +#define DB_DEFAULT_UART_BAUD_RATE 57600 #elif CONFIG_DB_GENERIC_BOARD

## Diff: main/db_led_indicator.c (Status LED)

--- a/main/db_led_indicator.c +++ b/main/db_led_indicator.c @@ -28,8 +28,14 @@ -#ifdef CONFIG_DB_OFFICIAL_BOARD_1_X_C6 +#if defined(CONFIG_DB_OFFICIAL_BOARD_1_X_C6) +#define DB_HAS_STATUS_LED 1 #define DB_STATUS_LED_GPIO GPIO_NUM_15 #define DB_STATUS_LED_ACTIVE_LOW 1 +#elif defined(CONFIG_DB_XIAO_ESP32C5_BOARD) +#define DB_HAS_STATUS_LED 1 +#define DB_STATUS_LED_GPIO GPIO_NUM_27 +#define DB_STATUS_LED_ACTIVE_LOW 1 +#else +#define DB_HAS_STATUS_LED 0 #endif

## Diff: main/main.c (Reset Pin & Wi-Fi Protocols)

--- a/main/main.c +++ b/main/main.c @@ -65,6 +65,8 @@ #elif CONFIG_IDF_TARGET_ESP32C6 #define DB_RESET_PIN GPIO_NUM_9 +#elif CONFIG_IDF_TARGET_ESP32C5 +#define DB_RESET_PIN GPIO_NUM_28 @@ -324,8 +326,25 @@ void db_init_wifi_apmode(int wifi_mode) { ESP_ERROR_CHECK(esp_wifi_set_mode(WIFI_MODE_AP)); +#if CONFIG_SOC_WIFI_SUPPORT_5G + if (wifi_mode == DB_WIFI_MODE_AP_LR) { + wifi_protocols_t protocols = { + .ghz_2g = WIFI_PROTOCOL_11B ¦ WIFI_PROTOCOL_LR, + .ghz_5g = 0 + }; + ESP_ERROR_CHECK(esp_wifi_set_protocols(WIFI_IF_AP, &protocols)); + } else { + wifi_protocols_t protocols = { + .ghz_2g = WIFI_PROTOCOL_11B ¦ WIFI_PROTOCOL_11G ¦ WIFI_PROTOCOL_11N ¦ WIFI_PROTOCOL_11AX, + .ghz_5g = WIFI_PROTOCOL_11A ¦ WIFI_PROTOCOL_11N ¦ WIFI_PROTOCOL_11AC ¦ WIFI_PROTOCOL_11AX + }; + ESP_ERROR_CHECK(esp_wifi_set_protocols(WIFI_IF_AP, &protocols)); + } +#else if (wifi_mode == DB_WIFI_MODE_AP_LR) { ESP_ERROR_CHECK(esp_wifi_set_protocol(WIFI_IF_AP, WIFI_PROTOCOL_11B ¦ WIFI_PROTOCOL_LR)); } else { ESP_ERROR_CHECK(esp_wifi_set_protocol(WIFI_IF_AP, WIFI_PROTOCOL_11B)); } +#endif

# 6. Operational Guide & Hardware Button Usage

## 6.1 Hardware Button Operations (Seeed Studio XIAO ESP32-C5)

The XIAO ESP32-C5 features two buttons adjacent to the U.FL antenna port:
• Button 'R' (Left): Hardware EN/Reset pin. Hard resets the power to the CPU.
• Button 'B' (Right / Underside, GPIO 28): Software rescue and mode toggle trigger:
   - Quick Click (< 350 ms): Instantly forces mode back to Access Point (AP), resets SSID to 'DroneBridge for ESP32', password to 'dronebridge', saves to NVS flash, and reboots.
   - Long Press (> 3 s): Performs a complete factory wipe and restores all default settings.

## 6.2 Switching between Access Point and Client Mode

• To switch from AP to Client mode: Connect to 'DroneBridge for ESP32' -> Open http://192.168.2.1 -> Under 'ESP32 Mode' select 'Wi-Fi Client Mode' -> Enter your Wi-Fi SSID & Password -> Click 'Save Settings & Reboot'.
• To switch from Client back to AP mode: Give a quick single click to hardware button 'B'.